# Metaswitch SIP Provisioning Server Endpoint Pack Documentation

This document explains how to use a specific endpoint pack with the Metaswitch SIP Provisioning Server. If you require further help using this pack and this is an accredited or certified device then Metaswitch will be able to provide assistance. For community packs, please contact the pack author.

For further details on how to develop or extend endpoint packs please see the official documentation, including “Creating an Endpoint Pack”. To find out more about your SIP end-point, please consult the Admin and User guides.

Note: this endpoint pack is only compatible with SIP PS SDK version 2.02 and above. It should not be installed on systems using earlier versions. If you are unsure which version of SIP PS SDK you are running, see https://communities.metaswitch.com/docs/DOC-91097 for a guide.

Please note: this pack requires EAS version 8.1.04-14 or above (inc. 8.3) to function. Installing it on earlier versions could cause phones not to register successfully.

For general information on how to use the SIP Provisioning Server, please contact your Metaswitch representative.

| ## Pack Details |  |
| --- | --- |
| Endpoint Pack ID | Yealink_T2X_certified |
| Author | Yealink Network Technology Co., Ltd |
| Minimum SIP PS Version | 2.02 |
| Version | 8 |
| Support contact | support@yealink.com |
| Date of Publication | 2017-12-08 |
| End-point Vendor | Yealink |
| End-point devices covered by this pack | T19P,T20P ,T21P,T21P_E2,T22P,T23P/G,T26P,T28P,T19_E2,T27G,T29G |
| Firmware download | http://support.yealink.com/documentFront/forwardToDocumentFrontDisplayPage?category=Desktop%20IP%20Phone&?BaseInfoCateId=1316&NewsCateId=1316&CateId=1313 |

Note: The firmware files are removed to reduce the EP size. If you need the firmware files, you can download them from Yealink website or contact Yealink support.

| ## Setup Instructions |  |
| --- | --- |
| DHCP Option to use | 66 |
| DHCP URL | http[s]://example.com/sip-ps/ |
| How to configure provisioning URL via web interface | - Navigate to the ‘Settings’ link on the upside. - Navigate to the ‘Auto Provision’ link on the left hand side - Enter http[s]://example.com/sip-ps/ in the Server URL field. - Click on”Confirm” to save |
| How to configure provisioning URL via endpoint menu | Menu >4. Settings > 2. Advanced Settings> 9.Auto Provision> 1.URL |
| Authorisation mode | HTTP Digest |
| Non-alphanumeric character input instructions (enter n/a if not possible to enter) | Press the 'Abc' softkey to switch to non-numeric input mode. Press key * for ＂*,!:@+-$ /＂，key # for ＂#＂ |

| ## Authentication Login |  |
| --- | --- |
| HTTP Digest login instructions | For T19P,T20P ,T21P,T22P,T26P,T28P： After phone starts,the screen will display "Zero touch" interface, click OK, then click Next, enter the subscriber DN as username and EAS password as password, and click OK. Note: If you do not see the "Zero touch" in the screen, please click Setting-> Auto Provision-> Zero Active,enable this feature,and restart the phone. For T21P_E2,T23P/G,T27G,T29G: After phone starts,the LCD screen will display a "Login" Soft Key when the phone is idle, click "Login" Soft Key, The phone will display an input form asking for a username and password. Enter the subscriber DN for the username and their EAS password for the password,and click OK. |

| ## Default Settings |  |
| --- | --- |
| Default user password for web UI | user |
| Default passwords for endpoint menu, if different from web UI |  |
| Default admin password for web UI | admin |

## Miscellaneous

This pack is compatible with Yealink firmware X.71.202.X , X.80.202.X and X.82.0.X.

## Changes to MWI defaults

Please note that the behaviour in the pack has now changed so that phones do not SUBSCRIBE to MWI by default. This is because the CFS Remote Media Gateway Model fixbit "Expects unsolicited MWI" is set by default in our recommended RMGMs. There are now two options:

- “Phone uses unsolicited MWI” (default): the phone does not send SUBSCRIBEs to MWI. If this is chosen the “Expects unsolicited MWI” fixbit should be checked in the Remote Media Gateway Model.

- “Phone subscribes to MWI”: the phone does send SUBSRIBEs to MWI. If this is chosen the “Expects unsolicited MWI” fixbit should be NOT be checked in the Remote Media Gateway Model. If the phone is a shared line then it will not be able to subscribe and you should use "Phone uses unsolicited MWI".

If you wish your phones to continue sending SUBSCRIBEs to MWI after installing the latest version of this pack, you will need to change the MWI setting to “Phone subscribes to MWI”.