# Metaswitch SIP Provisioning Server Endpoint Pack Documentation

This document explains how to use a specific endpoint pack with the Metaswitch SIP Provisioning Server. If you require further help using this pack and this is an accredited or certified device then Metaswitch will be able to provide assistance. For community packs, please contact the pack author.

For further details on how to develop or extend endpoint packs please see the official documentation, including “Creating an Endpoint Pack”. To find out more about your SIP end-point, please consult the Admin and User guides.

Note: this endpoint pack is only compatible with SIP PS SDK version 2.06 and above. It should not be installed on systems using earlier versions. If you are unsure which version of SIP PS SDK you are running, see https://communities.metaswitch.com/docs/DOC-91097 for a guide.

Please note: this pack requires EAS version 9.5 or above (inc. 9.5) to function. Installing it on earlier versions could cause phones not to register successfully.

For general information on how to use the SIP Provisioning Server, please contact your Metaswitch representative.

| ## Pack Details |  |
| --- | --- |
| Endpoint Pack ID | Yealink_T3X_certified |
| Author | Yealink Network Technology Co., Ltd |
| Minimum SIP PS Version | 2.06 |
| Version | 2 |
| Support contact | https://ticket.yealink.com |
| Date of Publication | 2020.12.01 |
| End-point Vendor | Yealink |
| End-point devices covered by this pack | T30,T30P,T31,T31P,T31G,T33P,T33G |
| Firmware version(s) included | T31(T30,T30P,T31G,T31P,T33P,T33G)- 124.85.0.35.rom |
| Firmware version(s) suggested | T31(T30,T30P,T31G,T31P,T33P,T33G)-124.85.0.35.rom and after |
| Firmware download | http://support.yealink.com/documentFront/forwardToDocumentFrontDisplayPage?category=Desktop%20IP%20Phone&?BaseInfoCateId=1316&NewsCateId=1316&CateId=1313 |

| ## Setup Instructions |  |
| --- | --- |
| DHCP Option to use | 66 |
| DHCP URL | http[s]://example.com/sip-ps/ |
| How to configure provisioning URL via web interface | - Navigate to the ‘Settings’ link on the upside. - Navigate to the ‘Auto Provision’ link on the left hand side - Enter http[s]://example.com/sip-ps/ in the Server URL field. - Click on”Confirm” to save |
| How to configure provisioning URL via endpoint menu | Settings > Advanced> Auto Provision> 1.URL |
| Authorisation mode | HTTP Digest |
| Non-alphanumeric character input instructions (enter n/a if not possible to enter) | Directly input with virtual keyboard. |

| ## Authentication Login |  |
| --- | --- |
| HTTP Digest login instructions | For now, the username and password can only be configured via web interface or endpoint menu beside “Provisiong URL”. There is no “Login” Softkey in the phone screen. And the phone will not popup any dialog box to ask for username and password. |

| ## Default Settings |  |
| --- | --- |
| Default user password for web UI | user |
| Default passwords for endpoint menu, if different from web UI |  |
| Default admin password for web UI | admin |

## Miscellaneous

This pack is compatible with Yealink firmware T31(T30,T30P,T31G,T31P,T33P,T33G)-124.85.0.15.rom.

## Release Notes

| Change | Comments | SFR (if applicable) |
| --- | --- | --- |
| Initial Release | Initial Release |  |